AIN SHAMS UNIVERSITY

FACULTY OF ENGINEERING

Manufacturing project

| NO. | Name | ID |
| --- | --- | --- |
| 1 | Eslam Mahmoud Mohamed | 1902235 |
| 2 | Ahmed Khaled khalaf | 1700059 |
| 3 | Mohamed Hassan Ragab | 1807892 |
| 4 | Ahmed Abdel karim Ahmed | 1804221 |
| 5 | Ziad Mohammed Helmy | 1700567 |
| 6 | Abdelhamed Abubakrv Abdelhamed | 18t6373 |
| 7 | Ahmed Atef Talaat | 1807933 |

Heavy Duty caster

Part 1

Base

NO. OFF 1

| Raw material | Before process |
| --- | --- |
| a=120 B=120 t=30 |  |

| Manufacturing process | After process |
| --- | --- |
| a-vertical milling 1-face milling with depth of cut 5mm 2-end milling with depth of cut 5mm twice 3-mack chamfer b-turning machine 1-facing with depth of cut 5mm and feed 40mm 2-facing to make D90 with depth of cut 4mm c-drilling machine 1-drill D20 2-drill 4D10 3-make counter sinking |  |

Part 2

Bush

No. off 1

| Raw material | Before process |
| --- | --- |
| D=40 l=200 |  |

| Manufacturing process | After process |
| --- | --- |
| 1-Facing 2-use central drill to located the center of w.p 3-reduce the diameter to 35mm then to 30mm 4-parting off with leaving 5mm added to demanding length for the facing of the other face so parting off after 55mm 5-drill a through hole with a diameter 20mm then diameter 25mm 6-facing the other side |  |

Part 3

Frame base

No. off 1

| Raw material | Before process |
| --- | --- |
| D=130 l=160 |  |

| Manufacturing process | After process |
| --- | --- |
| 1-Facing 2-use central drill to located the center of w.p 3-reduce the diameter to 105mm then to 100mm 4- parting off with leaving 5mm added to demanding length for the facing of the other face so parting off after 25mm 5- drill a through hole with a diameter 15mm then diameter 20mm 6-increase the inner diameter to 35mm and increase the depth to 5mm 7-repeat step(6) in the other side with change the inner diameter to 47mm 8-marking for shoot w.p from 2 sides 9-marking 4 centers for 4 holes 10-drilling 4 holes with depth 11mm and diameter 6mm 11- thread these holes(M6X1) |  |

Part 4

Frame lift side

No. off 1

| Raw material | Before process |
| --- | --- |
| a=90 b=130 t=15 |  |

| Manufacturing process | After process |
| --- | --- |
| a- milling machine 1-horyzontal milling step 1-depth of cut 4mm step 2-depth of cut 1mm step 3-depth of cut 2mm b-vertical milling machine 1-end milling with depth of cut 2.5 2-shering the slope 3-mack chamfer c-drilling machine 1-center drill 2-drilling D6 through all 3-drilling D25 through all |  |

Part 5

Frame right side

No. off 1

| Raw material | Before process |
| --- | --- |
| a=90 b=130 t=15 |  |

| Manufacturing process | After process |
| --- | --- |
| a-in milling machine 1-horyzontal milling step 1-depth of cut 4mm step 2-depth of cut 1mm step 3-depth of cut 2mm b-vertical milling machine 1-shering edges 2-shering the slope 3-mack chamfer c-drilling machine 1-center drill 2-drilling D6 through all 3-drilling D25 through all |  |

Part 9

King pin

No. off 1

| Raw material | Before process |
| --- | --- |
| D=40 l=200 |  |

| Manufacturing process | After process |
| --- | --- |
| 1-Facing 2- use central drill to located the center of w.p 3- reduce the diameter to 35mm then to 30mm 4- reduce the diameter to 25mm then to 20mm with leaving 5mm in diameter 30mm in the side 5-leave 3mm from the other side then reduce the diameter to 18mm with depth 2mm 6-Making one chamfer by tilting the cutting tool 45 7- parting off with leaving 5mm added to demanding length for the facing of the other face so parting off after 43mm 8- Facing the other face |  |

Part 11

Pin

No. off 1

| Raw material | Before process |
| --- | --- |
| D=40 l=200 |  |

| Manufacturing process | After process |
| --- | --- |
| a-turning machine 1-straight turning D35 to length 107mm(depth of cut 5mm) 2- straight turning D25 to length 92mm (depth of cut 5mm twice) 3- straight turning D20 to length 25mm (depth of cut 5mm twice) b-drilling machine 1-drill D4 c-vertical milling machine 1-sharing 2 chord in head of pin d-turning machine 1-mack the thread |  |

Part 15

Tire

No. off 1

| Raw material | Before process |
| --- | --- |
| D=130 l=160 |  |

| Manufacturing process | After process |
| --- | --- |
| a-turning machine 1-Facing 2- use central drill to located the center of W.P 3- 2-turning with depth of cut 2.5mm to make D120 and feed 34mm. Repeat this 2 times 4- parting off with leaving 5mm added to demanding length for the facing of the other face so parting off after 39mm 5-Drill through hole with any diameter 6-increase the inner diameter until reach to 100mm 7-Facing the other face |  |

Part 16

Wheel flange

No. off 2

| Raw material | Before process |
| --- | --- |
| D=130 l=160 |  |

| Manufacturing process | After process |
| --- | --- |
| a-turning machine 1-facing 2-turning with depth of cut 2.5mm to make D110 and feed 16mm. Repeat this 4 times 3-cross turning to make D94 with depth of cut 3 and feed 47 from center. Repeat this 4 times 4-turning before 3mm with depth of cut 5mm and feed 12mm 5-cut W.P b-drilling machine 1-centering all holes 2-drill D40 3-drill 6D6 |  |

Part 17

Wheel hub

No. off 1

| Raw material | Before process |
| --- | --- |
| D=130 l=160 |  |

| Manufacturing process | After process |
| --- | --- |
| a-turning machine 1-facing 2-turning with depth of cut 2mm and feed 50. Repeat this 7times. 3- turning with depth of cut 1mm and feed 50. To make D100 4- turning with depth of cut 3mm and feed 20. Repeat this 10 times. 5-do the same in another direction. 6-make fillet 7-drill D30 8-Bore the hole b-drilling machine 1-centering all holes 2-drill 6D6 |  |